CIVE 202 Report

Project 3 – Transportation Data Analysis & IDM Simulation

3/30/26

Contributor:

Rashid Hamid

# Table of Contents

Introduction	3

Methods	3

Results and Discussion	4

Conclusion	5

References	5

# Introduction

Client: The Federal Highway Administration (FHWA)

The Federal Highway Administration, also known as FHWA, has been collecting data on transportation to help in infrastructure, traffic, and safety. This data is important in understanding how people move and how they interact with their vehicles, especially under real life conditions. In this project, two types of data were analyzed: the National Household Travel Survey, which offered information on how people move and information on vehicles, and the Next Generation Simulation, which focused on vehicle movements and driving behavior. The goal of the project was to create charts and plots to design a simulation model to help in understanding transportation trends and vehicle interactions. The findings of the project will assist engineers in improving efficiency, safety, and planning in transportation.

# Methods

The analysis was performed using Python, along with pandas and matplotlib libraries. The variables were chosen based on their relevance to transportation engineering and their ability to show trends in the data. The variables were selected from the NHTS dataset, namely, travel_day, vehicle_age, and vehicle_type. The variable travel_day was used to compare frequency of trips on different days of the week, whereas vehicle_age was used to compare the age of vehicles. Vehicle_type was combined with vehicle_age to compare differences.

From the NGSIM data, the variables "Time," "leader_speed," "follower_speed," "leader_position," and "follower_position" were chosen for analysis. A single trajectory has been extracted for easier analysis.

Some visualizations were created. A bar chart for the trip frequency over the days of the week was made by using travel_day. A histogram for vehicle ages was made by using vehicle_age. A box plot for comparing the distributions of vehicle type and vehicle age was made by using vehicle_type and vehicle_age. Time series plots for the follower vehicle's response to the leader vehicle were made by using Time with speed variables. Spacing, which is the difference between the leader and follower vehicle positions, was plotted over time.

The Intelligent Driver Model (IDM) was implemented using a loop structure in Python, which simulates follower vehicle movement. The IDM model uses significant parameters, which include desired speed (v0), minimum spacing (s0), desired time headway (T), maximum acceleration (a), comfortable deceleration (b), and acceleration exponent (delta). The follower vehicle movement is determined by calculating acceleration based on relative speed and spacing, which are then used to update the follower vehicle's speed and position, by simulating a realistic movement like a real-life scenario.

# Results and Discussion

The bar chart of travel_day showed that the frequency of trips is much higher on weekdays compared to weekends. This means that the major factor affecting transportation demand is commuting. The information is useful in transportation planning because it reveals the peak hours of the day in which there is a high possibility of road congestion.

The analysis of the spacing showed how the distance between the vehicles is changing over time. This is very important for the overall safety of the road and the flow of the road itself.

The histogram for vehicle_age showed that most of the vehicles are of moderate age. This is important for the overall planning of the infrastructure of the road because the age of the vehicle plays an important role in the overall safety of the road.

The boxplot for vehicle_type and vehicle_age showed that some of the vehicle types have a wider age range than others. This is important for the overall analysis of the road and the overall usage of the road over a longer period.

The time-series plots showed that the speed of the follower vehicle is changing based on the leader vehicle. However, in real-world data, the speed of the follower vehicle is changing compared to the simulation data. This is because the real-world data is also changing due to the reaction time of the driver and the overall disturbances.

The spacing analysis demonstrated the pattern of how the spacing between vehicles varies over time. This is important in understanding the flow of traffic and safety. Proper spacing reduces accidents, and it affects the efficiency of the road.

The IDM simulation demonstrated a pattern close to the original data but did not account for all the variations that can be experienced in real driving. This demonstrates the limitations of simple models, which cannot fully account for human behavior. The limitations demonstrate the importance of adjustment and validation of models before they can be used in real-world scenarios.

# Conclusion

This project has successfully studied data related to transportation and created visualizations to identify major trends in travel patterns and interactions of vehicles. The implementation of the IDM has investigated simulated driving patterns and compared them with real data. Though the trends were correctly identified by the model, the variability in real-world situations could not be perfectly replicated by the model. The results obtained in this project can be effectively used in many projects related to transportation engineering.

# References

Federal Highway Administration (FHWA). National Household Travel Survey (NHTS).

Federal Highway Administration (FHWA). Next Generation Simulation (NGSIM) Dataset.